EV Highway Corridor Reliability Gap Analysis: Project Report

Author: Inbavathi

Geography: India (36 states and Union Territories)

Type: Exploratory data analysis and geospatial analysis (portfolio project)

Contents

- Executive summary

- Background and problem statement

- Objectives and analysis questions

- Data sources

- Data cleaning

- Methodology

- Results

- Robustness checks

- Recommendations

- Limitations

- Future work

- Appendix A: Parameters

- Appendix B: Output files

- Appendix C: Original plan versus what was built

1. Executive summary

India has registered about 10.4 million EVs between 2017 and 2026 (partial year), but only about 7% are cars or buses, the vehicles that depend on highway fast chargers. Public charger counts are usually reported per state, which can hide long highway stretches with no fast charger.

This project maps 1,822 operational public chargers from Open Charge Map onto India's national highway network and measures, for each ~25 km road segment, the distance to the nearest fast charger (≥ 25 kW). A segment is a gap when no fast charger is within 50 km.

Main results:

- 60% of the mapped road network (98,551 km) has no fast charger within 50 km.

- On six key highways, coverage ranges from 100% (NH-544) to 50% (NH-52). NH-44 has the longest uncovered run, 427 km.

- Charger count alone is misleading: NH-66 has 243 fast chargers within 5 km yet a 376 km uncovered stretch.

- State-level, Bihar, Chandigarh, Assam, Tripura and Punjab have the highest combined gap scores. Ten states/UTs have no mapped charger at all.

- Highway ratings do not change when the fast-charger definition switches between 25 kW and the dataset's own flag. Rankings of the state score are more sensitive to how the weights are chosen.

Because Open Charge Map is crowdsourced, some of the red areas reflect incomplete mapping. The results are best read as a priority list for verification and investment, not as a final audit.

2. Background and problem statement

Range anxiety is the fear of running out of charge before reaching a charger, and it is a common reason people hesitate to buy an EV. Cities are comparatively well served. Highways need chargers that are close enough together, fast enough to be useful, and actually working.

State totals can mislead. A state with hundreds of chargers concentrated in one city can still have a stretch of highway with none. A route-level view shows where the failure is.

Problem statement. Existing analyses mostly aggregate chargers by state or city. They do not show whether fast charging is available at sensible intervals along major highways. This project measures fast-charger coverage along selected national highways and compares charging supply with EV demand across states.

3. Objectives and analysis questions

Objectives

- Build clean, analysis-ready datasets for chargers, EV registrations, highways and state boundaries.

- Measure fast-charger coverage and gaps along India's road network and along six key highways.

- Compare charging supply with EV demand by state.

- Combine the evidence into an explainable state-level gap score and a highway scorecard.

- Test whether conclusions hold when assumptions change.

Questions answered in the notebook

| Q | Question |
| --- | --- |
| Q1 | How fast is EV adoption growing, and which states drive it? |
| Q2 | Where is EV demand high but fast-charger supply low (EV deserts)? |
| Q3 | How powerful are the chargers, and who runs them? |
| Q4 | How reliable is fast-charging coverage on each key highway? |
| Q5 | Where exactly are the gaps on the map? |
| Q6 | Which states should be prioritised for new fast chargers on the key highways? |
| Q7 | Do the conclusions change if the gap distance or "fast charger" definition changes? |

4. Data sources

| Dataset | Source | Coverage | Raw size |
| --- | --- | --- | --- |
| EV registrations: all classes | Vahan Dashboard | 36 states/UTs, calendar years 2017 to 2026, ELECTRIC(BOV) and PURE EV, status ACTIVE | 36 rows (Total row removed) |
| EV registrations: cars | Vahan Dashboard, Category Group = Four Wheeler | Same filters | 36 rows |
| EV registrations: buses | Vahan Dashboard, Category Group = Bus | Same filters | 36 rows |
| Charging stations | Open Charge Map API, country code IN | Station location, operator, status, max power, number of points, dates | 1,979 rows, 15 fields kept |
| National highways | MoRTH via PM GatiShakti, from India Geodata (updated 30-06-2022) | Highway lines with road name, type, status, lanes, traffic fields | 10,317 rows, 65 columns |
| State and UT boundaries | India Geodata, States and Union Territories | Polygons for 36 states/UTs | 36 rows, 13 columns |

Why cars and buses separately. Two-wheelers and e-rickshaws dominate total registrations but rarely use highway fast chargers. The all-classes file gives overall adoption, while cars plus buses (hw_ev) is the demand measure for highway charging.

Licensing note. Open Charge Map data is CC BY 4.0 and needs attribution. The GatiShakti highway data is released under CC0 on the India Geodata release page.

5. Data cleaning

Each dataset was cleaned in its own notebook. The checks and fixes below are exactly what was run.

5.1 State and UT boundaries

| Step | Result |
| --- | --- |
| Missing values, duplicate rows, duplicate state names | None found |
| Invalid geometries | 3 found and repaired with make_valid |
| Geometry types | 20 Polygon, 16 MultiPolygon (all valid after repair) |
| Columns kept and renamed | state, state_code, lgd_code, geometry, plus a lowercase state_key |
| Name standardisation | Title case, & → and, extra spaces removed, two names corrected (Dadra and Nagar Haveli and Daman and Diu; Andaman and Nicobar Islands) |
| CRS check | OGC:CRS84. Bounds fall within India's extent |
| Overlap check | No overlaps larger than 1 km² between states |
| Area | Computed in EPSG:7755. Total about 3.18 million km², inside the expected 3.0 to 3.6 million range |
| Multipart geometry | Andaman and Nicobar Islands has 606 parts. Many are very small islands, which is expected |

Saved as states_clean.parquet (36 rows).

5.2 EV registrations (all, car, bus)

The same checks ran on all three files.

| Step | Result |
| --- | --- |
| Header text in first column | Renamed to State |
| Total row | Removed (37 → 36 rows) |
| Duplicates, blanks, whitespace, missing values, negative values | None |
| Yearly values vs Total column | Zero mismatches in all three files |
| Zero values | Kept. They are real zeros for small states (for example Sikkim had 0 EV registrations until 2024 in the all-classes file) |

Saved as Ev_registration_cleaned.csv, Ev_registration_car_cleaned.csv, Ev_registration_bus_cleaned.csv.

Note: 2026 is a partial year, so it is drawn as a dashed line in trend charts.

5.3 Charging stations (Open Charge Map)

The raw API response was flattened into one row per station. Maximum power, total points and a fast-charging flag were derived from the nested connector list.

| Step | Rows removed | Rows left |
| --- | --- | --- |
| Raw API pull | n/a | 1,979 |
| Invalid or missing state after cleaning | 37 | 1,942 |
| Power above 500 kW (unrealistic, maximum seen was 1,000,000) | 2 | 1,940 |
| Duplicate exact coordinates | 97 | 1,843 |
| Coordinates outside India (ID 301542, labelled Erode, Tamil Nadu) | 1 | 1,842 |
| Implausible record: 600 points at 0 kW with unknown town (ID 479330) | 1 | 1,841 |

Other fixes:

- State names: about 30 spelling variants were mapped to a standard list, including typos ("Keraka", "Lerala", "Maharshtra"), abbreviations ("GJ", "KA", "MH", "MP", "RJ") and cities entered as states ("Ahmedabad", "Burari", "Villupuram"). "India" was treated as invalid.

- Missing text fields: town, postcode and operator were filled with Unknown / Unknown Operator. Missing power was set to 0 so the station location is kept.

- Operator labels: (Unknown Operator) merged into Unknown Operator.

- Kept on purpose: six Bengaluru hubs with 56 to 136 points (genuine large hubs) and two Channapatna stations at almost the same spot (different operators).

- Consistency checks: no duplicate IDs, and the status text agrees with the is_operational flag for every row.

Saved as Ev_chargers_cleaned.csv (1,841 rows).

5.4 National highways

| Step | Result |
| --- | --- |
| Columns | 65 reduced to 29 relevant ones (state, road name and type, lane status, completion, lengths, traffic fields, classification levels, status, geometry) |
| Redundant traffic columns | traffic_2 (only "Data Not Available" or 0) and traffic_13 (identical to traffic_12) dropped |
| Traffic values | "Data Not Available" → missing, then converted to numbers. No negatives |
| Text fixes | Madhya Pradesh spelling variants merged. Road name "Delete" set to missing. Lane status "IL" set to missing and spacing variants of 2L/4L/6L/2LPS standardised |
| Lengths | Source length_2 and gis_length contain zeros (137 rows with both zero). A new length_km was calculated from the geometry in EPSG:6933 |
| Exact duplicate rows | 64 removed (10,317 → 10,253) |
| Duplicate geometries | 746 removed, keeping the row with the fewest missing key attributes (10,253 → 9,507) |
| Geometry checks | No missing, empty or invalid geometries. 9,253 LineString and 254 MultiLineString |

Saved as highways_clean.geojson (9,507 rows, 28 columns). Road types in the cleaned file: National Highway 9,276, State Expressway 93, Other Major Road 67, State Highway 64, Frontier Highway 7.

6. Methodology

6.1 Parameters

| Parameter | Value | Meaning |
| --- | --- | --- |
| METRIC_CRS | EPSG:7755 | Projected CRS in metres for distance and length |
| FAST_KW | 25 kW | Operational chargers at or above this power count as fast |
| CORRIDOR_KM | 5 km | A charger within 5 km of a highway belongs to that corridor |
| SEG_KM | 25 km | Target segment length |
| GAP_KM | 50 km | A segment is a gap when its nearest fast charger is farther than this |
| MIN_COMPONENT_KM | 5 km | Ignore very short highway fragments |
| MIN_DEMAND | 5,000 | Minimum car + bus EVs for a state to appear in the EV-desert ranking |
| Target highways | NH-44, NH-48, NH-16, NH-66, NH-544, NH-52 | Six highways studied in detail |

6.2 Charger preparation

- Keep chargers marked operational and set 0 kW to missing.

- Define is_fast as max_power_kw >= 25. The dataset flag was compared and not used, because it disagrees with power for 152 stations (53 flagged fast below 25 kW, 99 not flagged fast at 25 kW or more).

- Convert to points and project to EPSG:7755.

- Assign each charger to a state with a spatial join. Unmatched points snap to the nearest state within 25 km. Result: 1,822 operational chargers, 1,421 fast.

6.3 State master table

Demand and supply were combined per state:

- Demand: total EVs, EV growth rate 2021 to 2025 (CAGR), electric cars, electric buses, hw_ev = cars + buses.

- Supply: operational chargers, fast chargers, total charging points, median power.

- Derived: chargers per 1,000 km², EVs per charger, evs_per_fast_charger = hw_ev / (fast chargers + 1), fast chargers per 100,000 highway-using EVs.

- EV desert flag: hw_ev at or above the median and fast chargers below the median.

6.4 National road network gap detection

- Project highway lines, split multi-part lines, keep line geometries and drop duplicate geometries (11,231 lines, 163,239 km).

- Split each line into segments of about 25 km (11,046 segments after removing pieces under 0.5 km).

- Take each segment midpoint, assign it to the nearest state and find the distance to the nearest fast charger using a k-d tree.

- Flag the segment as a gap when that distance is more than 50 km.

6.5 Six-highway corridor analysis

- Merge each target highway into continuous lines and drop fragments under 5 km. Lengths before and after de-duplication:

| Highway | Raw summed km | Unique km | Kept km | Pieces kept |
| --- | --- | --- | --- | --- |
| NH-16 | 2,238.9 | 1,750.0 | 1,711.8 | 25 |
| NH-44 | 3,967.4 | 3,591.6 | 3,558.1 | 34 |
| NH-48 | 2,679.8 | 2,555.0 | 2,542.5 | 24 |
| NH-52 | 2,319.8 | 2,232.4 | 2,228.7 | 23 |
| NH-544 | 326.5 | 330.6 | 330.6 | 2 |
| NH-66 | 2,049.0 | 1,711.6 | 1,646.0 | 43 |

Summing the raw rows overstates length because the source file repeats stretches, so merged unique length is used.

- Build a 5 km buffer around each highway. Chargers inside a buffer belong to that highway (878 unique chargers near any of the six).

- Cut each highway into ~25 km segments (519 segments in total).

- For each segment, measure the distance to the nearest fast charger on the same highway. A segment is a gap when this exceeds 50 km. 161 of 519 segments are gaps.

- Assign each segment to a state by its midpoint.

6.6 Highway scorecard

For each highway: length, number of fast chargers in the corridor, fast chargers per 100 km, uncovered km, covered % = 100 × (1 − uncovered km / road km), and longest gap = the longest run of consecutive gap segments.

| Rating | Covered % |
| --- | --- |
| Reliable | above 90 |
| Patchy | above 70 up to 90 |
| Poor | 70 or below |

6.7 State gap score (EV Desert Index)

Three indicators, each min-max scaled to 0 to 1 where 1 is worse:

| Component | Definition | Weight |
| --- | --- | --- |
| Demand pressure | scaled log of (EVs per charger) | 0.40 |
| Highway gap | scaled % of the state's highway length in gap | 0.35 |
| Charger sparsity | scaled log of chargers per 1,000 km², inverted | 0.25 |

Gap score = 100 × (0.40 × demand + 0.35 × highway gap + 0.25 × density). States are ranked, and split into Low, Medium and High gap classes using terciles. The weights are the analyst's judgement and are tested in Section 8.

Two points to know when reading the table: states with no highway in the data (Chandigarh and Lakshadweep) use the median highway-gap value, and a state with zero chargers is treated as having one charger when computing EVs per charger.

6.8 Clustering

K-Means (k = 3, random state 42) on the three scaled components after standardisation, used as a supporting view.

7. Results

7.1 EV adoption (Q1)

- Total registrations across all classes: 10,377,820 (2017 to 2026, 2026 partial).

- Cars and buses make up about 7% of all registrations.

- Largest totals by state: Uttar Pradesh 1,815,492; Maharashtra 1,170,473; Karnataka 912,484; Tamil Nadu 708,760; Rajasthan 569,096.

7.2 Charger network (Q3)

- 1,822 operational chargers, 1,421 fast (≥ 25 kW).

- 27.8% of operational chargers have an unknown operator.

- 86% of operational chargers lie within 5 km of a mapped highway.

- Mapped chargers are concentrated: after cleaning, Kerala (596), Karnataka (378), Odisha (284) and Tamil Nadu (211) lead the list of states, while Bihar, Punjab, Assam and Jharkhand have 1 or 2 each.

- Ten states/UTs have zero mapped chargers: Andaman and Nicobar Islands, Arunachal Pradesh, Ladakh, Lakshadweep, Manipur, Meghalaya, Mizoram, Nagaland, Sikkim, Tripura.

7.3 EV deserts (Q2)

Among states with at least 5,000 electric cars and buses, the highest ratio of cars + buses per operational fast charger is in Telangana (about 5,500), Punjab (about 3,960) and Delhi (about 3,780). Chandigarh follows at about 3,640. Punjab and Chandigarh are also flagged by the quadrant rule (high EV count, few fast chargers).

Reading note: Open Charge Map is crowdsourced, so very low counts partly reflect incomplete mapping.

7.4 National network gaps

98,551 km, or 60% of the mapped road network, has no fast charger within 50 km (6,582 of 11,046 segments). The map shows red concentrated in the north and east, which partly reflects thin charger mapping there.

7.5 Six-highway scorecard (Q4, Q5)

| Highway | Length (km) | Fast chargers | Per 100 km | Covered % | Uncovered km | Longest gap (km) | Rating |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NH-544 | 330.6 | 101 | 30.6 | 100.0 | 0.0 | 0.0 | Reliable |
| NH-48 | 2,542.5 | 191 | 7.5 | 97.7 | 58.2 | 27.1 | Reliable |
| NH-66 | 1,646.0 | 243 | 14.8 | 70.1 | 491.8 | 376.1 | Patchy |
| NH-16 | 1,711.8 | 38 | 2.2 | 64.4 | 609.0 | 126.2 | Poor |
| NH-44 | 3,558.1 | 186 | 5.2 | 56.0 | 1,566.8 | 426.6 | Poor |
| NH-52 | 2,228.7 | 19 | 0.9 | 50.0 | 1,114.7 | 173.4 | Poor |

Observations:

- NH-544 is fully covered and has the densest fast charging (30.6 per 100 km), but it is only 330.6 km long.

- NH-48 is long (2,543 km) and still 97.7% covered, so long routes can be well served.

- NH-66 has the most nearby fast chargers (243) yet a 376 km uncovered run. The chargers are clustered rather than spread along the route. NH-66 sits just above the 70% line, so its "Patchy" label is a narrow call.

- NH-44 is the longest highway studied and has the single longest uncovered run (427 km) and the most uncovered length (1,567 km).

- NH-52 has the fewest fast chargers (19) and the lowest coverage (50%).

"Longest gap" here is the longest run of consecutive segments whose midpoints are more than 50 km from a fast charger. The actual driving distance between the two chargers at either end of such a run is larger than the run itself.

7.6 State priority on the six highways (Q6)

States ranked by highway length with no fast charger within 50 km (top 12):

| State | Highway km studied | Gap km | Gap % | Car + bus EVs | Fast chargers in state |
| --- | --- | --- | --- | --- | --- |
| Maharashtra | 1,833.8 | 891.2 | 48.6 | 117,168 | 75 |
| Madhya Pradesh | 895.4 | 795.9 | 88.9 | 16,643 | 11 |
| Andhra Pradesh | 1,309.4 | 453.3 | 34.6 | 20,752 | 14 |
| Punjab | 335.8 | 335.8 | 100.0 | 11,873 | 2 |
| Rajasthan | 1,306.9 | 327.8 | 25.1 | 42,182 | 46 |
| Uttar Pradesh | 314.7 | 248.9 | 79.1 | 43,468 | 20 |
| Jammu and Kashmir | 360.0 | 186.4 | 51.8 | 1,958 | 1 |
| West Bengal | 186.1 | 186.1 | 100.0 | 20,032 | 7 |
| Telangana | 497.8 | 175.2 | 35.2 | 60,455 | 10 |
| Haryana | 398.6 | 165.2 | 41.4 | 15,486 | 26 |
| Gujarat | 501.0 | 51.8 | 10.3 | 47,893 | 36 |
| Karnataka | 1,479.9 | 23.0 | 1.6 | 79,943 | 323 |

Maharashtra has both the largest uncovered length and the largest car + bus EV base among the top gaps, which makes it the clearest case for new fast chargers. Punjab and West Bengal have their whole studied length uncovered, but each covers only a short stretch of the six highways.

7.7 State gap score (EV Desert Index)

Top 12 states, which form the High gap class:

| Rank | State | EVs (all) | Chargers | EVs per charger | Highway in gap % | Gap score |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Bihar | 560,438 | 1 | 560,438 | 100.0 | 75.1 |
| 2 | Chandigarh | 27,419 | 1 | 27,419 | 72.2 (median used) | 70.9 |
| 3 | Assam | 384,300 | 2 | 192,150 | 100.0 | 70.9 |
| 4 | Tripura | 45,221 | 0 | 45,221 | 100.0 | 64.8 |
| 5 | Punjab | 181,928 | 2 | 90,964 | 85.7 | 62.9 |
| 6 | Uttar Pradesh | 1,815,492 | 28 | 64,839 | 76.3 | 58.8 |
| 7 | Madhya Pradesh | 513,003 | 13 | 39,462 | 78.6 | 57.1 |
| 8 | West Bengal | 341,644 | 9 | 37,960 | 75.4 | 56.2 |
| 9 | Jharkhand | 139,850 | 6 | 23,308 | 79.8 | 55.6 |
| 10 | Manipur | 4,115 | 0 | 4,115 | 100.0 | 55.0 |
| 11 | Jammu and Kashmir | 54,215 | 1 | 54,215 | 69.0 | 54.8 |
| 12 | Uttarakhand | 114,889 | 14 | 8,206 | 86.0 | 54.8 |

Chandigarh has no highway in the data, so its highway-gap component is the median of other states and should be read with caution.

Clusters (K-Means, k = 3):

| Cluster | States | Mean gap score | Mean EVs | Mean chargers |
| --- | --- | --- | --- | --- |
| 0 | 18 | 52.4 | 408,849 | 23.0 |
| 1 | 9 | 37.9 | 334,214 | 156.4 |
| 2 | 9 | 44.5 | 1,179 | 0.0 |

Cluster 0 is the large group with substantial EV adoption and few chargers. Cluster 1 has the most chargers and the lowest gap score. Cluster 2 is small-EV states and UTs with essentially no mapped chargers.

8. Robustness checks (Q7)

8.1 Gap distance rule: % of highway covered

| Highway | 25 km | 50 km | 75 km | 100 km |
| --- | --- | --- | --- | --- |
| NH-16 | 43.6 | 64.4 | 77.5 | 86.4 |
| NH-44 | 39.7 | 56.0 | 65.9 | 74.4 |
| NH-48 | 81.6 | 97.7 | 99.7 | 99.7 |
| NH-52 | 31.4 | 50.0 | 59.0 | 68.0 |
| NH-544 | 100.0 | 100.0 | 100.0 | 100.0 |
| NH-66 | 65.8 | 70.1 | 74.6 | 76.1 |

The order of highways is almost unchanged. NH-544 and NH-48 stay at the top and NH-52 and NH-44 stay at the bottom. Even at a generous 100 km, NH-52 is 68% covered.

8.2 Definition of "fast charger": % of highway covered

| Highway | ≥ 22 kW | ≥ 25 kW | ≥ 50 kW | Dataset flag |
| --- | --- | --- | --- | --- |
| NH-16 | 64.4 | 64.4 | 33.3 | 24.2 |
| NH-44 | 56.0 | 56.0 | 48.0 | 56.0 |
| NH-48 | 97.7 | 97.7 | 81.7 | 95.6 |
| NH-52 | 50.0 | 50.0 | 28.9 | 50.0 |
| NH-544 | 100.0 | 100.0 | 100.0 | 100.0 |
| NH-66 | 70.1 | 70.1 | 70.1 | 70.1 |

Ratings are the same under the two realistic definitions (≥ 25 kW and the dataset flag) for every highway. Two things move: NH-48 drops from Reliable to Patchy at ≥ 50 kW, and NH-16 is very sensitive, ranging from 24% to 64% depending on the definition. NH-16 should therefore be treated as clearly weaker than NH-48 but with an uncertain exact figure.

8.3 State score weights

| Scenario | Top-10 overlap with base | Rank correlation with base |
| --- | --- | --- |
| Base 0.40 / 0.35 / 0.25 | 10 | 1.000 |
| Equal weights | 8 | 0.964 |
| Demand-heavy (0.6 / 0.2 / 0.2) | 8 | 0.885 |
| Highway-heavy (0.2 / 0.6 / 0.2) | 6 | 0.816 |
| Density-heavy (0.2 / 0.2 / 0.6) | 5 | 0.446 |
| Demand as cars + buses per fast charger | 8 | 0.896 |

The ranking is fairly stable for equal and demand-heavy weights. It changes a lot when charger density dominates, so the exact order of the top states depends on the weights chosen. The group of high-gap states is more reliable than any single rank.

9. Recommendations

These follow from the results above and assume the data limits in Section 10.

- Prioritise NH-44, NH-52 and NH-16 for fast-charger additions or verification. They rate Poor under every realistic definition tested.

- Fix NH-66 by spacing, not by adding more chargers. It already has 243 fast chargers nearby, but a 376 km uncovered stretch. Chargers should go into the empty stretch.

- Start with Maharashtra and Madhya Pradesh on the key highways. They have the largest uncovered lengths (891 km and 796 km). Maharashtra also has the biggest car and bus EV base among the top gaps.

- Treat Bihar, Assam, Punjab and Tripura as high-gap states. They combine many EVs (or high growth) with very few mapped chargers. Confirm with ground data, because mapping is thin in these states.

- Use NH-544 and NH-48 as reference models of how dense, evenly spread fast charging looks.

- For any single recommendation, verify against another charger source (operator maps or official state counts) before committing money.

10. Limitations

- Crowdsourced supply data. Open Charge Map can miss stations and keep outdated records. Gaps in thinly mapped states may be partly mapping gaps.

- Status is a snapshot. Only the operational flag is used. True uptime, queues and breakdowns are not measured. Records were not filtered by how recently they were verified.

- Unknown operators. 27.8% of operational chargers have no operator listed.

- Straight-line distance. Segment midpoints are measured to the nearest fast charger in a straight line. Driving distance and charger direction of travel are not modelled.

- Highway data date and status. The highway file was last updated 30-06-2022, and the analysis network is not filtered by road status, so proposed and under-construction lines in the source file are included. The national gap figure covers all road types in the file, not national highways alone.

- Analyst choices. The 25 kW fast definition, the 5 km buffer, the 25 km segment length and the 50 km gap rule are chosen thresholds, not official standards. Sensitivity checks cover the gap distance and fast definition, but not the buffer width or segment length.

- Demand proxy. Vahan registrations are by state, not by route, and 2026 is year-to-date. Registered vehicles are not the same as vehicles travelling on a given highway.

- Weights are a judgement. The state score weights are not derived from data and the ranking is sensitive to them (Section 8.3).

- State assignment. Highway segments are assigned to the nearest state by midpoint, so segments near borders can be assigned to a neighbour.

- Small-state artefacts. States with no highway in the data use a median value, and states with no chargers rely on a floor of one charger in the EVs-per-charger ratio.

- Six highways only. Corridor-level conclusions apply to NH-44, NH-48, NH-16, NH-66, NH-544 and NH-52 and cannot be generalised to other routes.

11. Future work

- Collect charger snapshots over time to estimate reliability instead of presence.

- Use along-road distance and direction of travel for gaps.

- Add traffic volume and more corridors, or the full national highway network.

- Test buffer width and segment length in the sensitivity checks.

- Cross-check Open Charge Map totals against official state-wise charger counts.

- Model the minimum number of new sites needed to bring every gap under a chosen distance.

- Build an interactive dashboard (for example Power BI) or web map for route-level lookup.

- Add SQL versions of the corridor summary and gap queries.

12. Appendix A: Parameters

| Constant | Value |
| --- | --- |
| METRIC_CRS | EPSG:7755 |
| CORRIDOR_KM | 5 |
| SEG_KM | 25 |
| GAP_KM | 50 |
| MIN_COMPONENT_KM | 5 |
| MIN_DEMAND | 5000 |
| FAST_KW | 25 |
| USE_DATASET_FLAG | False |
| TARGET_HIGHWAYS | NH-44, NH-48, NH-16, NH-66, NH-544, NH-52 |
| YEARS | 2017 to 2026 |
| State score weights | demand 0.40, highway gap 0.35, density 0.25 |

13. Appendix B: Output files

Cleaned data (data/Cleaned datasets/): states_clean.parquet, highways_clean.geojson, Ev_registration_cleaned.csv, Ev_registration_car_cleaned.csv, Ev_registration_bus_cleaned.csv, Ev_chargers_cleaned.csv.

Tables (outputs/tables/)

| File | Content |
| --- | --- |
| highway_scorecard.csv | Six-highway scorecard |
| state_priority_6_highways.csv | Uncovered km on the six highways by state |
| highway_rating_by_definition.csv | Ratings under each fast-charger definition |
| state_gap_master.csv | State master table with gap score and class |
| national_segments_gap.geojson | National road segments with gap flag |
| segments_6_highways_gap.geojson | Six-highway segments with gap flag |
| chargers_clean.geojson | Operational chargers with state and highway distance |

Figures (outputs/figures/): 20 charts and maps, numbered 01_ev_growth to 20_score_vs_corridor_gap.

14. Appendix C: Original plan versus what was built

The project started from a broader plan and was narrowed to what could be completed and defended.

| Topic | Original plan | What was built |
| --- | --- | --- |
| Routes | 8 to 12 city-pair corridors | Six national highways analysed as full corridors |
| Corridor buffer | About 3 km | 5 km |
| Gap definition | Distance between consecutive fast chargers along the route | Distance from each ~25 km segment midpoint to the nearest fast charger, flagged above 50 km |
| Fast charger threshold | 30 kW | 25 kW (plus 22 kW, 50 kW and dataset flag in sensitivity checks) |
| Score | Four-part corridor score including operational share and stale-record share | State-level EV Desert Index (demand, highway gap, density) plus a highway scorecard |
| Outputs | Dashboard, SQL queries, one-page business summary | Notebooks, 20 figures, CSV and GeoJSON tables, this report |

Items from the plan that are not part of the current results: operational-share and stale-record metrics per corridor, a Power BI dashboard, and SQL scripts. They are listed under Future work.